# VISTA — Software Requirements Specification

# Problem and practical application

VISTA supports operators arranging light objects on a table: pick a cube or pen and place it completely in a box. The verified proof-of-concept is safe bimanual teleoperation, dataset collection, simulation/sim-data, SmolVLA fine-tuning and safety-gated inference.

# Scope

Local-first control host owns USB serial to two SO-101 followers. Web, phone and VR are operator surfaces. Observation uses two wrist and two overview head cameras. Internet control, RBAC, ACT and Diffusion Policy are out of scope.

# Acceptance

Dataset: 150 real episodes, split 105/15/30. Timeout over one second causes Hold. E-stop locks motion; recovery requires reconnect, recenter and preflight. SmolVLA must pass at least 7 of 10 cube trials and 7 of 10 pen trials without safety violation.

# Traceability

See docs/requirements/01-registration-extract.md through 12-traceability-matrix.md. Existing evidence: VR teleop video 7307978483558391289.mp4 and dual-follower video 8689445351821956196.mp4.

# Approval and open items

GVHD must approve scope changes in registration v1.1. Dataset manifest, simulation run, model evaluation and timeout/E-stop evidence remain required before final acceptance.